- बाढी, डुवान सम्बद्ध खोज तथा उद्धार सामाग्रीहरु

| SN | Items | Specification |
| --- | --- | --- |
| १ | Rescue Raft | - Raft Specifications: - Length:(16 to 17) feet - Width:(7 to 7.7) feet - Weight :(170 to 188) lbs. - Frame Width:(66 to 72) inch - Frame Length:(85 to 96) inch - Tube Diameter: (20 to 22) inch - Thwart System: Min. 4 Removable (Batten Attachment Thwart system) - Thwart Diameter: (12to 14) inch - Center Compartment: Width (43 to 48) inch - Kick\Rocker (Bow\Stern): (30to 32) inch - Number of Air Chambers: Min.9 - Valve Type: Lea field C-7 & Pressure Relief - Weight/Denier of Tube Fabric: 48/1670 – 50/1672 - Weight/Denier of Floor Fabric: 48/1670-50/1672 - Number of D-Rings: Min. 14 - Number of Handles: Min.8 - Material: Heavy-duty Hypalon, 100% Pennel Orca material - Color: Red/Orange - Self-Bailing system floor Minimum Ten-year guaranty with fabric of raft - Accessories (in each raft) - Boat Bag - Boat Repair kit Includes- (5 set in each raft) - Plastic Container - Patching Piece- 2 roll - Valve -2 - Valve cover -2 - Dendrite- 1 liter - Hardener- 1liter - Rubbing Stake Folder- 2 - Roller (1 small, 1 big) - Emergency Tear Aid Patch- 2 - Alcohol Patch- 2 - Pump valve Nozzle- 2 - Key-2 - Life line - Lifting handle - Super D-ring (4 units extra in each set) Drain plug (Pressure Release Valve) set 8 extras - Other Requirements - Certified and Tested document - Guaranty Certificate document of company - Raft repairing and maintenance practical five days training for thirty personnel with all necessary logistic management - Color: Orange/Red Luminous Print “OF ORGANIZATION” on boat (both side, front and back side |
| २ | Paddle Per Boat | - Outfitter Paddle (Twelve set in each raft) - Grip: T-grip - Total length: (58 to 60) inch - Blade Size: Width (7.5 to 8) inch, Length (20 to 22) inch - Blade Shape: Symmetrical - Blade Material: Xenoy - Shaft Material- Polypropylene -covered aluminum - Shaft Shape: Straight - Breakdown: No - Weight: (1000 to 1080) g - Minimum three-year guaranty |
|  |  | - Command (Guide)Paddle (Two set in each raft) - Grip: T-grip - Total length: (64 to 66) Inch - Blade Size: Width (7.5 to 8) inch, Length (25-27) inch - Blade Shape: Symmetrical - Blade Material: Xenoy - Shaft Material: Polypropylene-covered aluminum - Shaft Shape: Straight - Breakdown: No - Weight: (1300- 1400) gram - Minimum three-year guaranty |
| ३ | Barrel Pump (two set in each raft) | - Action- inflate - Ump dimension- Min. Diameter 6-inch, Min. Length 24 inch - Volume per stroke- (2 - 2.03) gal. - Hose length- (6.5 to 7) feet - Weight-(11 – 13) lbs. - Features- wide base and carry handle Made of heavy-duty PVC, Foam O-ring seal - Minimum ten-year guaranty |
| ४ | Rescue Throw Bag | - Weight: Max. 2 lb 4 oz - Rope Length: Min. 75' - Rope Material: Polypropylene yellow for high visibility. - Rope Diameter: Min. 3/8" - Load Max: 1900 lbs. (8.45 kN) - Features: Mesh on top of bag for quicker drying, Webbing loop for glow stick - Rugged Cordura Bag with two high visibility colors. - 2" mesh panel for drainage - An elastic loop and 'pocket' with a glow stick or "chem light"An elastic loop and 'pocket' with a glow stick or "chem light" - Certified and Tested document - Guaranty Certificate document of company |
| ५ | Back Board | - Lightweight, foam-filled polyethylene backboard with a smooth surface for easy patient transfer. - Runners allow easier access to the handholds for raising and lowering the board. - Compatible with most head immobilization devices. - Can be worn during x-ray procedures. - Suitable for water rescues. - Includes 21 pins for use with speed clip restraining straps. - Capacity: Min. 273 kg (600 lbs) - Dimensions: 45.1 x 185 x 5.7 cm (17-3/4" x 72-3/4" x 2-1/4"); - Weight: Max. 7.3 kg (16 lbs) - Included Restraint Straps w/Swivel Speed Clip & Automotive-Type Buckle - Four-piece restraint straps made of 5.1 cm (2") nylon webbing with a quick-release push button buckle (automobile-type) and metal swivel speed clip - For use with spinal backboard with pins - Dimensions: 5.1 x 152.4 cm (2" x 5') - Included Universal Foot Immobilizer w/Straps - Included Universal Head Immobilizer w/Straps - Immobilization device for use with an extrication collar when head or neck injuries are suspected - Features include: side blocks with wide fast-stick closures for a snug fit and up to 50% more support surface; ear holes to monitor bleeding and fluid drainage; 2 head/chin straps - Positions on a spinal backboard with mounting straps - No metal components - can be worn during x-rays, MRI and CT scan procedures - Waterproof and buoyant - can be used for water rescues - Lightweight and easy to use; can be disinfected for reuse - Weight: Max. 0.9 kg (2.1 lbs) - Included Extrication Collar - Adjustable Sizes - Features comprehensive sizing system and application for a perfect fit on patients, ranging from neckless (size 3) to tall (size 6) - Rigid one-piece construction - no assembly required - Radiolucent, CT and MRI compatible - No post to break - Extra-large trachea opening - Flip chin piece allows for intubations and airway management - Large genuine Velcro closures - Folds completely flat for storage - Minimum five-year warranty - Certified and Tested document - Guaranty Certificate document of company |
| ६ | Carabineer (Screw lock) | - Shape: D- shape/oval-shape - Weight: Max. 270 g. - Gate Opening: (20 to 30) mm - Vertical(major) axis strength: Min. 40 KN - Horizontal (Minor) axis strength: (15-17) KN - Open gate strength: Min. 11 KN - Screw Lock - Material: Aluminum Alloy/ Steel - Certification(s): CE EN 362, EAC, NFPA - Minimum five-years guaranty - Certified and Tested document - Guaranty Certificate document of company |
|  | Figure 8 Carabineer | - Strength: Min. 30 KN - Rope diameter compatibility:(7.5 to 13) mm - Weight: Max. 106 g. - Material: Aluminum Alloy/ Steel - Certification(s): CE EN, EAC, NFPA - Minimum five-year guaranty - Certified and Tested document - Guaranty Certificate document of company |
| ७ | 500 mm petrol Chain Saw | - Engine Displacement (cu.in. /cc): Min. 4.8 cu.in. (78.5 cc) - Power Rating: Min. 5.7 HP/4.3 kW - Max. Engine Speed: Min. 12,800 RPM - Chain Saw Configuration: Powerhead only - Pitch: Min. 3/8" - Fuel Mixture: 50:1 - Fuel Tank Capacity: Min. 25.4 oz. - Oil Tank Capacity: Min. 14.2 oz. - Net Weight (w/o bar & chain): Max. 14.7 lbs. - User Type: Professional - Chain Saw Feature: Heavy duty air filter - Power Type: 2-stroke gas - Chain Selection: Min. 3/8” x .050 - Weight: Max. 18.5 lbs. - Guide Bar: 20” - Chain cover - 20” Saw Chain 5 set in each set of equipment - Tools kit set - In each set of equipment Sparking Plug and filter set five/five pcs same model and brand as in chain saw set - Heavy-duty air filter system - Easy-starting automatic half throttle lock - Chain compartment - Two-point durable inertia/mechanical chain brake - Decompression valve - Lateral chain tensioning - Large metal spike bar - Minimum Five years guaranty - Certified and Tested document - Guaranty Certificate document of company |
| ८ | Static Rope | - Diameter: (11to 13) mm - Elongation50/150 Kg.: 2.9% min. - Weight: 70- 95 grams per meter - Static Strength: Min. 38 KN - Protected in use against mechanical damages (abrasion, cutting, chafing, etc.) - Sheath slippage: 0 % - Breaking load: 35kN min. - Sheath: (38 – 43) % - Weight of the core: 57% min. - Shrinkage H2O: (3 -4) % - Length: 100 m. per coil - Material: PA (Polyamide) - Certification: NFPA, EN, NFPA, UIAA - Manufacture date not before 2018 - Minimum Five Years Guaranty - Certified and Tested document - Guaranty Certificate document of company |
|  | Dynamic Rope | - Diameter: (11 to 13) mm - Static Elongation: 7.5% min. - Static Elongation: 31% min - Weight: (70- 85)grms per meter - Breaking load: 40 kN min. - Impact Force:7.9KN Minimum - Protected in use against mechanical damages (abrasion, cutting, chafing, etc.) - Sheath slippage: 0 % - Length: 100 m. per coil - Material: PA (Polyamide) - Certification: NFPA, EN, NFPA, UIAA - Minimum Five Years Guaranty - Manufacture date not before 2018 - Certified and Tested document - Guaranty Certificate document of company |

- आगलागी नियन्त्रण सम्बद्ध खोज तथा उद्धार सामाग्रीहरु

.  Fire Fighting Suit

(a)	 Fire Fighting Jacket

- Size: (86 to 94) cm chest wide

- Body height: (164 to 172) cm

- Outer fabric materials: NOMEX Tough, approx. 195 g/m3

- Moisture barriers liner water and windproof, breathable

- Safety thermal lining with insulating layer in a 100% aramid nonwoven and aramid/ viscose quilting, approx. 270 g/m3

- Arm and jacket with moisture blocker made of flame retardant PU coated aramid

- Sleeve construction with mobility wedge

- Additional reinforcement at elbows and shoulders made of silicone coated paraaramid fabric

- Flame protectionCollar

- Sleeve width with adjustable to hook and pile fastener

- Sleeve with NOMEX knitted wristlet with thumb hole

- Radio pocket both side

- Two side Pocket with flap

- Lamp holder on front cover

- Adjusting Cord around waist

- Loops at shoulders patches

- (15 x 2.5) cm fleece strip for attaching name strip above left radio pocket

- 5 cm wide silver and Two 1.5 cm wide orange reflective stripes around torso and both sleeve

- Backside length: (85 to 90) cm

- Color: Dark Blue

- Weight: (2 to 2.5) kg

- Certification: EN 469

- Minimum ten-year guaranty

- Luminous Print APF back side of jacket

(b)  Fire Fighting Trousers

- Size: (82 to 84) cm Waist

- Body height:(164 to 172) cm

- Certification: EN 469

- Outer fabric: NOMEX

- Trouser with adjustable, non-detachable approx. 5 cm wide suspenders

- Adjustable elastic straps for fixing with waist band individually

- Fly with zipper and hook and pile fastener, covered by flap

- Two side pockets with flap

- Trousers with moisture blocker of flame retardant PU coated aramid

- Reinforcement at the knees, on the cuffs and interior seams of pants made of flame retardant silicon-carbon coated paraara-mid fabric

- Fix mounted padding under the knee protection

- 5 cm wide silver and two 1.5 cm wide orange reflective strips around legs

- Weight: (1.5 to 1.8) kg

- Color: Dark Blue

- Minimum ten-year guaranty

(c)  Helmet(following must be in each set)

- Certification: EN443, PR EN 16471/3, ISO 16073

- Helmet type: Full shell

- Size adjustment form outside

- Size:(51 to 65) cm

- Face Shield: As per EN 14458

- Eye Protection visor

- Neck protector

- Mask adaptor with spring clamp

- All around neck protector

- Edge protector with leather chin guard

- Integrated torch holder,

- Helmet Torch System:  LEDexplosion proof with AAA batteries with Holster

- Integrated mask adaptor

- Radio headset

- Comfort Padding chin strap

- Leather lining

- Heat radiation: Min. 8 minutes, 14 KW/m2 ≥ 3000C helmet temp.

- Cleaning spray: 5 per set

- Shock absorption: Hemispherical striker, 5 kg foam a height of 2.5 m, in heated condition > 3000Chelmet temp., Force transmitted to heat < 15 KN

- Penetration resistance: Flat blade striker, 1kg foam height of 2 m, in heated condition > 3000C helmet temp.

- Flame engulfment: 1000 0C/10 sec, no after burning >5 sec, no dripping

- Helmet carrying bag

- Color: Orange/Red/Yellow

- Luminous Print APF on two side of helmet

- Minimum ten-year guaranty

(d) Fire-fighting gloves

- Certification: EN 659, Gore-Tex X-TRAFIT technology

- Plame side: silicon carbon mineral coated

- NOMEX KEVLAR double face fabric

- Flexible wristlet adjustment

- Pleats on the finger

- Adjustable hook and pile fastening

- Polyurwthane protector on hands’ back with heat insulation

- Cuff with reflective stripes

- Abrasion, Cut, Tear, Puncture resistant EN 388

- Heat resistant EN 367 ISO 6942

- Contact heat resistant EN 702

- Shrinkage ISO 17493

- Dexterity EN 420

- Water permeability EN ISO 20344

- Fluid permeability ISO 15383

- Chemical permeability EN 368

- Color: Dark blue

- Minimum ten-year guaranty

(e) Fire-fighting Boot

- Certification: EN 15090

- Ankle protector

- Hydrophobic waterproof leather

- Lined with Sympatex climate membrane

- High grade laminate lining with foamed material, highly abrasion resistance

- 3M reflective material, silver made of heat-retardant material

- Ankle-Zone stiffening

- Joint supportive leather heel insert

- Steel toe cap and steel midsole

- Leather insole

- Nitrile outsole

- Rubber toe cap

- BOA lacing system

- Reflex-element as sight able areas

- Extreme low weight

- Size: (41 to 43) cmEU

- Height: 28 cm approx.

- Weight: 2.7 kg max. (1 pair)

- Replacement coiler and lace: 2/2 per pairs

- Minimum ten-year guaranty

(f)         Motor Pump

- Engine: 4 stroke gasoline

- Connection: minimum two tools

- Operation: Two tools min.

- Turbo function

- Power: Min. 4.2 KW

- Electric starter

- Max. operating pressure: 700 bars

- Flow rate, low presser- high pressure: 2 x 3.1 – 2 x 0.9 l/min

- Flow rate turbo, low pressure- high pressure: 1 x 6.05- 1 x 1.7 l/min

- Oil quantity: Min. (4.8to 5) ltr.

- Dimension:  Max. (485 x 440 x 445) mm, (810 x 440 x 590) mm (with hose reel)

- Weight: Max. 42 kg, 80.5 kg (with hose reel)

- Hose length: Min. (2 x 20) m (with hose reel)

- Certification: EN 13204, NFPA 1936

- Fire Blanket

- Fire Blanket is 66" x 82"

- Dupont X-12 fire retardancy.

- Made of aramide

- Aluminum ladder /extension ladder 3 (section)

- rope pull with rope break

- 2 supports bars

- two spindles for leveling purpose

- wall rubber castle

- automatic ladder lock

- lifting capacity-324 kg

- length-14 mtr